IT5437: COMPUTER VISION

Assignment 02: Fitting and Alignment

Name: Manuel I.T

Index Number: 258799R

GitHub Link:

Question 01

Part (a) requires fitting a line using Total Least Squares (TLS).

Unlike standard regression, TLS minimizes the orthogonal distance from points to the line, providing a more balanced fit when noise exists in both axes.

1.1 Total Least Squares (Part a)

The first line parameters were extracted using Singular Value Decomposition (SVD) on centered data. The smallest singular value corresponds to the normal vector of the best-fit line.

Resulting Parameters:

a = 0.3162, b = -0.9487, d = -11.3841

Line Equation: 0.3162x - 0.9487y - 11.3841 = 0

This segment shows how you used Singular Value Decomposition (SVD) to find the line parameters.

Part (b) Sequencial RANSAC

To handle multiple lines in a single dataset, a sequential RANSAC algorithm was implemented. In each iteration, RANSAC finds the largest consensus set. As per the assignment hint, once a line is identified, its inliers are "masked" or removed from the dataset to allow the next iteration to find the subsequent line.

This segment is crucial for part (b) because it shows how you followed the assignment "Hint" to run RANSAC multiple times and mask the consensus set (remove inliers) to find the next line

1.3 Discussion

While TLS is effective for a single line with balanced noise, it is highly sensitive to outliers. RANSAC proved essential for the combined dataset as it robustly ignored points from Line 2 and Line 3 while fitting Line 1. The sequential masking approach successfully isolated all three models without interference.

Question 02

2.1 Calculation and Magnification

Using the provided,

focal length = f = 8 mm

distance = d = 720 mm

magnification (M) is determined:

M = f / (d - f) = 8 / (720 - 8) = 8 / 712 ≈ 0.011236

Actual Diameter = (Pixel Radius × 2 × 0.0022 mm) / M

2.3 Discussion

The results show an Outer Diameter of 74.80 mm and an Inner Diameter of 35.15 mm. The accuracy of this measurement relies on the camera's optical axis being perpendicular to the earrings; any perspective tilt would introduce error into the diameter estimation.

Question 03

3.1 Manual Homography Calculation

This segment shows the process of taking the 6 manually clicked points and computing the transformation matrix.

3.2 Automated SIFT and RANSAC  Registration

This is the most technically significant part of the question. It demonstrates feature detection, descriptor matching, and the use of RANSAC to find a robust homography.

3.3 Results Comparison

| Method | Wrapped Image Result | Difference Image Result |
| --- | --- | --- |
| Manual(6 Points) | Visible "starburst" distortion near corners. | High residual noise due to misalignment. |
| SIFT + RANSAC | Clean, perspective-correct alignment. | Mostly black, highlighting only true changes. |

Figure 3.1: Warped Image - Manual Selection (6 Points)	       Figure 3.2: Warped Image - Automated SIFT + RANSAC

Figure 3.3: Difference Image - Automated SIFT + RANSAC.   Figure 3.4: Difference Image - manual

3.2 Discussion and Comparison

The manual method (Part a) failed to produce a clean registration because human point selection lacks sub-pixel accuracy. Small errors in the 6 manual points lead to significant perspective warping errors. In contrast, the automated SIFT method (Part c) utilized hundreds of feature points. Combined with RANSAC to discard outlier matches, it achieved a near-perfect alignment. The difference image for the automated method effectively isolates only the differences between the boards, proving the images are of the same object under different perspectives.